Safety Data Sheet

CONTACT CLEAN 200ML


[image: image1.png]Distaccanti Industriali
Confezione Prodotti in Aerosol - Spray




Safety Data Sheet dated 11/1/2023, version 5

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Mixture identification:

Trade name: 
CONTACT CLEAN 200ML

Trade code: 
A01029

UFI:
Q080-103E-G00R-6PSW

1.2. Relevant identified uses of the substance or mixture and uses advised against

Recommended use:

AGENT OF SURFACE DEGREASER

Uses advised against: 

do not use on humans and animals

do not use for purposes other than those indicated.

1.3. Details of the supplier of the safety data sheet

Company:

STAC PLASTIC SPRAY S.R.L. Via E. De Nicola 9/11 10036 Settimo Torinese (To) Italia.

STAC PLASTIC SPRAY S.R.L. Tel. n.. +39 011 8977566 Fax n. +39 011 8977491

Competent person responsible for the safety data sheet:

Massimiliano Brescia stacplas@stacplastic.com

1.4. Emergency telephone number

Centro Antiveleni di Milano 02 66101029 (CAV Ospedale Niguarda Ca’ Granda -Milano) 

Centro Antiveleni di Pavia 0382 24444 (CAV IRCCS Fondazione Maugeri - Pavia) 

Centro Antiveleni di Bergamo 800 883300 (CAV Ospedali Riuniti - Bergamo) 

Centro Antiveleni di Firenze 055 7947819 (CAV Ospedale Careggi - Firenze) 

Centro Antiveleni di Roma 06 3054343 (CAV Policlinico Gemelli - Roma) 

Centro Antiveleni di Roma 06 49978000 (CAV Policlinico Umberto I - Roma) 

Centro Antiveleni di Napoli 081 7472870 (CAV Ospedale Cardarelli - Napoli)

Centro Antiveleni di Verona 800 011858 (CAV Ospedale AOUI- Verona)

Centro Antiveleni di Foggia 800 183459 (CAV Policlinico Riuniti-Foggia)

Centro Antiveleni di Roma 06 68593726(CAV Ospedale Bambino Gesù-Roma)

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

EC regulation criteria 1272/2008 (CLP):

[image: image2.png]


  Danger, Aerosols 1, Extremely flammable aerosol. Pressurized container: may burst if heated.


[image: image3.png]


  Warning, Skin Irrit. 2, Causes skin irritation.


[image: image4.png]


  Warning, STOT SE 3, May cause drowsiness or dizziness.


[image: image5.png]


  Aquatic Chronic 2, Toxic to aquatic life with long lasting effects.


Adverse physicochemical, human health and environmental effects: 

No other hazards

2.2. Label elements

Hazard pictograms:

[image: image6.png]


[image: image7.png]


[image: image8.png]


 

Danger

Hazard statements:

H222, H229 Extremely flammable aerosol. Pressurized container: may burst if heated.

H315 Causes skin irritation.

H336 May cause drowsiness or dizziness.

H411 Toxic to aquatic life with long lasting effects.

Precautionary statements:

P101 If medical advice is needed, have product container or label at hand.

P102 Keep out of reach of children.

P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking.

P211 Do not spray on an open flame or other ignition source.

P251 Do not pierce or burn, even after use.

P271 Use only outdoors or in a well-ventilated area.

P273 Avoid release to the environment.

P410+P412 Protect from sunlight. Do not expose to temperatures exceeding 50 °C/122°F.

P501 Dispose of container in accordance with national regulation.

Special Provisions:

None

Contains

hydrocarbons c7, n-alkanes, isoalkanes, cyclics

propan-2-ol;  isopropyl alcohol;  isopropanol

Special provisions according to Annex XVII of REACH and subsequent amendments:

None

2.3. Other hazards

No PBT, vPvB or endocrine disruptor substances present in concentration >= 0.1%

Other Hazards:

No other hazards

SECTION 3: Composition/information on ingredients

3.1. Substances

N.A.

3.2. Mixtures

Hazardous components within the meaning of the CLP regulation and related classification:

	Qty
	Name
	Ident. Number
	Classification

	>= 40% - < 50%
	hydrocarbons c7, n-alkanes, isoalkanes, cyclics
	CAS: 

64742-49-0 

EC: 

927-510-4 

REACH No.: 

01-2119666169-27-0000 


	[image: image9.png]


 2.6/2 Flam. Liq. 2 H225 
[image: image10.png]


 3.2/2 Skin Irrit. 2 H315 
[image: image11.png]


 4.1/C2 Aquatic Chronic 2 H411 
[image: image12.png]/@\



 3.10/1 Asp. Tox. 1 H304 
[image: image13.png]


 3.8/3 STOT SE 3 H336

	>= 30% - < 40%
	propane
	Index number: 

601-003-00-5 

CAS: 

74-98-6 

EC: 

200-827-9 

REACH No.: 

01-2119486944-21-0046 


	[image: image14.png]


 2.2/1 Flam. Gas 1 H220 
[image: image15.png]\\f/



 2.5 Press. Gas H280

	>= 15% - < 20%
	Hydrocarbons, C4;  Petroleum gas
	Index number: 

649-113-00-2 

CAS: 

87741-01-3 

EC: 

289-339-5 

REACH No.: 

01-2119480480-41-xxxx 


	[image: image16.png]\\f/



 2.5 Press. Gas H280 
[image: image17.png]


 2.2/1 Flam. Gas 1 H220 
DECLK (CLP)*

	>= 5% - < 7%
	propan-2-ol;  isopropyl alcohol;  isopropanol
	Index number: 

603-117-00-0 

CAS: 

67-63-0 

EC: 

200-661-7 

REACH No.: 

01-2119457558-25-xxxx 


	[image: image18.png]


 2.6/2 Flam. Liq. 2 H225 
[image: image19.png]


 3.3/2 Eye Irrit. 2 H319 
[image: image20.png]


 3.8/3 STOT SE 3 H336

	>= 0.25% - < 0.5%
	2-butoxyethanol; ethylene glycol monobutyl ether
	Index number: 

603-014-00-0 

CAS: 

111-76-2 

EC: 

203-905-0 

REACH No.: 

01-2119475108-36-XXXX 


	[image: image21.png]


 3.1/4/Inhal Acute Tox. 4 H332 
[image: image22.png]


 3.1/4/Oral Acute Tox. 4 H302 
[image: image23.png]


 3.2/2 Skin Irrit. 2 H315 
[image: image24.png]


 3.3/2 Eye Irrit. 2 H319 
Acute Toxicity Estimate: 
ATE - Oral 1200 mg/kg bw


*DECLK (CLP): This substance is classified in accordance with Note K, Annex VI of EC Regulation CE 1272/2008. The harmonised classification as a carcinogen or mutagen applies unless it can be shown that the substance contains less than 0,1 % w/w 1,3- butadiene (Einecs No 203-450-8), in which case a classification in accordance with Title II of this Regulation shall be performed also for those hazard classes. Where the substance is not classified as a carcinogen or mutagen, at least the precautionary statements (P102-)P210-P403 shall apply.

SECTION 4: First aid measures

4.1. Description of first aid measures

In case of skin contact:

Immediately take off all contaminated clothing.

Areas of the body that have - or are only even suspected of having - come into contact with the product must be rinsed immediately with plenty of running water and possibly with soap.

Wash thoroughly the body (shower or bath).

Remove contaminated clothing immediately and dispose off safely.

After contact with skin, wash immediately with soap and plenty of water.

In case of  eyes contact:

After contact with the eyes, rinse with water with the eyelids open for a sufficient length of time, then consult an opthalmologist immediately.

Protect uninjured eye.

In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

In case of Ingestion:

Do not under any circumstances induce vomiting. OBTAIN A MEDICAL EXAMINATION IMMEDIATELY.

In case of Inhalation:

Remove casualty to fresh air and keep warm and at rest.

4.2. Most important symptoms and effects, both acute and delayed

For symptoms and effects due to the contained substances see chapter 11

4.3. Indication of any immediate medical attention and special treatment needed

In case of accident or unwellness, seek medical advice immediately (show directions for use or safety data sheet if possible).

Treatment: 

Follow the doctor's instructions.

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media:

CO2 or Dry chemical fire extinguisher.

In case of fire, use a foam fire extinguisher to extinguish.

Extinguishing media which must not be used for safety reasons:

Water.

5.2. Special hazards arising from the substance or mixture

Burning produces heavy smoke.

Do not inhale explosion and combustion gases.

5.3. Advice for firefighters

Use suitable breathing apparatus .

Collect contaminated fire extinguishing water separately. This must not be discharged into drains.

Keep containers cool with water spray.

Move undamaged containers from immediate hazard area if it can be done safely.

Normal elements for fire fighting, such as a self-contained compressed air open-circuit respirator (EN 137), fire-retardant suit (EN469), flame-retardant gloves (EN 659) and fire boots (HO A29 or A30).

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

Wear personal protection equipment.

Remove all sources of ignition.

Remove persons to safety.

See protective measures under point 7 and 8.

6.2. Environmental precautions

Do not allow to enter into soil/subsoil. Do not allow to enter into surface water or drains.

In case of gas escape or of entry into waterways, soil or drains, inform the responsible authorities.

Suitable material for taking up: absorbing material, organic, sand

Retain contaminated washing water and dispose it.

6.3. Methods and material for containment and cleaning up

For containment:

Limit in case of leakage of significant quantities of product. Contain the spread of small quantities of product with earth, sand or other inert absorbent material.

For cleaning up:

Wash with plenty of water.

Wet clean or vacuum up solids.

Clear spills immediately

Other information:

Don't use a brush or compressed air for cleaning surfaces or clothing.

6.4. Reference to other sections

See also section 8 and 13

SECTION 7: Handling and storage

7.1. Precautions for safe handling

Avoid contact with skin and eyes, inhalation of vapours and mists.

Do not use on extensive surface areas in premises where there are occupants.

Don't use empty container before they have been cleaned.

Before making transfer operations, assure that there aren't any incompatible material residuals in the containers.

See also section 8 for recommended protective equipment.

Use localized ventilation system.

Advice on general occupational hygiene:

Do not eat, drink or smoke when using this product.

Wash hands after use

Contamined clothing should be changed before entering eating areas.

7.2. Conditions for safe storage, including any incompatibilities

store in a cool, well ventilated place, away from heat, flames, sparks or other sources of ignition

keep only in the original container away from sunlight neighborhoods

avoid contact with skin and eyes, inhalation of vapours/mists/dusts. 

do not use empty containers before they are cleaned. 

contaminated clothing must be replaced before entering the dining areas. 

at work do not eat or drink.

avoid the accumulation of electrostatic charges.

do not smoke

Store at below 50 °C. Keep away from unguarded flame and heat sources.  Avoid direct exposure to sunlight.

Keep away from unguarded flame, sparks, and heat sources.  Avoid direct exposure to sunlight.

Keep away from food, drink and feed.

Incompatible materials:

Keep away from acids.

Keep away from water or from damp surroundings.

See subsection 10.5

Instructions as regards storage premises:

Cool and adequately ventilated.

Safety electric system.

7.3. Specific end use(s)

None in particular

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

hydrocarbons c7, n-alkanes, isoalkanes, cyclics - CAS: 64742-49-0


 TLV-ACGIH - Notes: TLV TWA - 400ppm-1639,26 mg/m3 (ACGIH) 

TLV STEL - 500ppm-2049,08 mg/m3 (ACGIH)

propane - CAS: 74-98-6


 ACGIH

propan-2-ol;  isopropyl alcohol;  isopropanol - CAS: 67-63-0


 ACGIH - TWA(8h): 200 ppm - STEL: 400 ppm

2-butoxyethanol; ethylene glycol monobutyl ether - CAS: 111-76-2


 EU - TWA(8h): 98 mg/m3, 20 ppm - STEL: 246 mg/m3, 50 ppm


 ACGIH - TWA(8h): 20 ppm

DNEL Exposure Limit Values

hydrocarbons c7, n-alkanes, isoalkanes, cyclics - CAS: 64742-49-0

Worker Professional: 300 mg/kg/d - Exposure: Human Dermal - Frequency: Long Term, systemic effects

Worker Professional: 2085 mg/l - Exposure: Human Inhalation - Frequency: Long Term, systemic effects

Consumer: 149 mg/kg/d - Exposure: Human Oral - Frequency: Long Term, systemic effects

propan-2-ol;  isopropyl alcohol;  isopropanol - CAS: 67-63-0

Consumer: 319 mg/kg/d - Exposure: Human Dermal - Frequency: Long Term (repeated)

Consumer: 89 mg/m3 - Exposure: Human Inhalation - Frequency: Long Term (repeated)

Consumer: 26 mg/kg/d - Exposure: Human Oral - Frequency: Long Term (repeated)

Consumer: 888 mg/kg/d - Exposure: Human Dermal - Frequency: Long Term, systemic effects

Consumer: 500 mg/m3 - Exposure: Human Inhalation - Frequency: Long Term, systemic effects

PNEC Exposure Limit Values

propan-2-ol;  isopropyl alcohol;  isopropanol - CAS: 67-63-0

Target: Fresh Water - Value: 140.9 mg/l

Target: Marine water - Value: 140.9 mg/l

Target: Freshwater sediments - Value: 552 mg/kg

Target: Marine water sediments - Value: 552 mg/kg

Target: Soil (agricultural) - Value: 28 mg/kg

8.2. Exposure controls

Eye protection:

Eye glasses with side protection. EN 166

Protection for skin:

Wear work clothes with long sleeves and protective footwear for professional use of category II (ref.Directive 89/686 / CEE and norm EN ISO 20344). Wash with soap and water after removing protective clothing.

Protection for hands:

Protect hands with category II work gloves (ref. Directive 89/686 / EEC and standard EN 374). Use PVC or nitrile rubber gloves.

Respiratory protection:

if the TLV thresholds are exceeded, use a mask with filter type A (against vapors of organic compounds) in accordance with EN 141.

Thermal Hazards:

Do not expose to temperatures exceeding 50° c.

Environmental exposure controls:

do not dispose of the product in the environment

emissions from production processes, including those from ventilation equipment should be inspected for the purposes of enforcement of environmental protection

Appropriate engineering controls:

None

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

	Properties
	Value
	Method:
	Notes:

	Physical state:
	Liquid
	--
	--

	Colour:
	Colourless
	--
	--

	Odour:
	sweet of solvent
	--
	--

	Melting point/freezing point:
	Not Relevant
	--
	--

	Boiling point or initial boiling point and boiling range:
	Not Relevant
	--
	--

	Flammability:
	N.A.
	--
	--

	Lower and upper explosion limit:
	Not Relevant
	--
	--

	Flash point:
	< 0 ° C
	--
	--

	Auto-ignition temperature:
	>175°C
	--
	--

	Decomposition temperature:
	Not Relevant
	--
	--

	pH:
	Not Relevant
	--
	--

	Kinematic viscosity:
	N.A.
	--
	--

	Solubility in water:
	Not Relevant
	--
	--

	Solubility in oil:
	Not Relevant
	--
	--

	Partition coefficient n-octanol/water (log value):
	Not Relevant
	--
	--

	Vapour pressure:
	4 bar +/- 1 20°C
	--
	--

	Density and/or relative density:
	0.675 +/- 0.05 kg/l
	--
	--

	Relative vapour density:
	> 2
	--
	--


Particle characteristics:

	Particle size:
	N.A.
	--
	--


9.2. Other information

No other relevant information

SECTION 10: Stability and reactivity

10.1. Reactivity

avoid contact with strong acids and bases and oxidizing agents.

10.2. Chemical stability

Stable under normal conditions

10.3. Possibility of hazardous reactions

None

10.4. Conditions to avoid

avoid the accumulation of electrostatic charges.

keep away from heat, sources of ignition

10.5. Incompatible materials

oxidizing agents

acids, alkalis and alkaline metals

10.6. Hazardous decomposition products

by thermal decomposition can rid COx

during combustion it produces irritating gases

SECTION 11: Toxicological information

11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

Toxicological information of the product:

CONTACT CLEAN 200ML

a) acute toxicity

Not classified

Based on available data, the classification criteria are not met

b) skin corrosion/irritation

The product is classified: Skin Irrit. 2 H315

c) serious eye damage/irritation

Not classified

Based on available data, the classification criteria are not met

d) respiratory or skin sensitisation

Not classified

Based on available data, the classification criteria are not met

e) germ cell mutagenicity

Not classified

Based on available data, the classification criteria are not met

f) carcinogenicity

Not classified

Based on available data, the classification criteria are not met

g) reproductive toxicity

Not classified

Based on available data, the classification criteria are not met

h) STOT-single exposure

The product is classified: STOT SE 3 H336

i) STOT-repeated exposure

Not classified

Based on available data, the classification criteria are not met

j) aspiration hazard

Not classified

Based on available data, the classification criteria are not met

Toxicological information of the main substances found in the product:

hydrocarbons c7, n-alkanes, isoalkanes, cyclics - CAS: 64742-49-0

a) acute toxicity:

Test: LC50 - Route: Inhalation - Species: Rat > 23300 mg/m3 - Duration: 4h

Test: LD50 - Route: Skin - Species: Rat > 2920 mg/kg

Test: LD50 - Route: Oral - Species: Rat > 5840 mg/kg

propan-2-ol;  isopropyl alcohol;  isopropanol - CAS: 67-63-0

a) acute toxicity:

Test: LD50 - Route: Oral - Species: Rat > 5840 mg/kg

Test: LD50 - Route: Skin - Species: Rabbit > 2000 mg/kg

2-butoxyethanol; ethylene glycol monobutyl ether - CAS: 111-76-2

a) acute toxicity

ATE - Oral 1200 mg/kg bw

11.2. Information on other hazards

Endocrine disrupting properties:

No endocrine disruptor substances present in concentration >= 0.1%

SECTION 12: Ecological information

12.1. Toxicity

Adopt good working practices, so that the product is not released into the environment.

CONTACT CLEAN 200ML

The product is classified: Aquatic Chronic 2 - H411

hydrocarbons c7, n-alkanes, isoalkanes, cyclics - CAS: 64742-49-0

a) Aquatic acute toxicity:

Endpoint: EC50 - Species: Fish = 1.5 mg/l - Duration h: 48

Endpoint: LC50 - Species: Daphnia = 4 mg/l - Duration h: 24

propan-2-ol;  isopropyl alcohol;  isopropanol - CAS: 67-63-0

a) Aquatic acute toxicity:

Endpoint: EC50 - Species: Daphnia > 100 mg/l - Duration h: 48

Endpoint: EC50 - Species: Algae > 100 mg/l - Duration h: 72

Endpoint: EC50 - Species: Fish > 100 mg/l - Duration h: 48

12.2. Persistence and degradability

None

N.A.

12.3. Bioaccumulative potential

N.A.

12.4. Mobility in soil

N.A.

12.5. Results of PBT and vPvB assessment

vPvB Substances: None - PBT Substances: None

12.6. Endocrine disrupting properties

No endocrine disruptor substances present in concentration >= 0.1%

12.7. Other adverse effects

None

SECTION 13: Disposal considerations

13.1. Waste treatment methods

Recover, if possible.  Send to authorised disposal plants or for incineration under controlled conditions.  In so doing, comply with the local and national regulations currently in force.

Additional disposal information:

contaminated packaging should be sent for recovery or disposal in compliance with national regulations on waste management

reuse if possible. Product residues are to be considered hazardous waste. disposal must be entrusted to authorised waste management, in compliance with national and, where appropriate, local.

CER 160504

SECTION 14: Transport information

14.1. UN number or ID number

ADR-UN number:
1950

IATA-Un number:
1950

IMDG-Un number:
1950

14.2. UN proper shipping name

ADR-Shipping Name:
AEROSOLS INFIAMMABILI

IATA-Technical name:
FLAMMABLE AEROSOLS 

IMDG-Technical name:
FLAMMABLE AEROSOLS 

14.3. Transport hazard class(es) 

ADR-Class:
2.5°F CAP. 2.2.2.1.6 UN1950

IATA-Class:
2.1

IMDG-Class:
2 Aerosols UN 1950

14.4. Packing group

ADR-Packing Group:
N.A. 

IATA-Packing group:
N.A.

IMDG-Packing group:
N.A.

14.5. Environmental hazards

Marine pollutant:
Marine pollutant

14.6. Special precautions for user

ADR-Transport category (Tunnel restriction code):
D

IMDG-Technical name:
FLAMMABLE AEROSOLS 

14.7. Maritime transport in bulk according to IMO instruments

Product is not allowed to be transported in bulk.

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

Dir. 98/24/EC (Risks related to chemical agents at work)

Dir. 2000/39/EC (Occupational exposure limit values)

Regulation (EC) n. 1907/2006 (REACH)

Regulation (EC) n. 1272/2008 (CLP)

Regulation (EC) n. 790/2009 (ATP 1 CLP) and (EU) n. 758/2013

Regulation (EU) n. 2020/878

Regulation (EU) n. 286/2011 (ATP 2 CLP)

Regulation (EU) n. 618/2012 (ATP 3 CLP)

Regulation (EU) n. 487/2013 (ATP 4 CLP)

Regulation (EU) n. 944/2013 (ATP 5 CLP)

Regulation (EU) n. 605/2014 (ATP 6 CLP)

Regulation (EU) n. 2015/1221 (ATP 7 CLP)

Regulation (EU) n. 2016/918 (ATP 8 CLP)

Regulation (EU) n. 2016/1179 (ATP 9 CLP)

Regulation (EU) n. 2017/776 (ATP 10 CLP)

Regulation (EU) n. 2018/669 (ATP 11 CLP)

Regulation (EU) n. 2018/1480 (ATP 13 CLP)

Regulation (EU) n. 2019/521 (ATP 12 CLP)

Regulation (EU) n. 2020/217 (ATP 14 CLP)

Regulation (EU) n. 2020/1182 (ATP 15 CLP)

Regulation (EU) n. 2021/643 (ATP 16 CLP)

Restrictions related to the product or the substances contained according to Annex XVII Regulation (EC) 1907/2006 (REACH) and subsequent modifications:

None

Where applicable, refer to the following regulatory provisions :

Directive 2012/18/EU (Seveso III)

Regulation (EC) nr  648/2004 (detergents).

Dir. 2004/42/EC (VOC directive)

Provisions related to directive EU 2012/18 (Seveso III):

Seveso III category according to Annex 1, part 1

Product belongs to category: P3a, E2

15.2. Chemical safety assessment

No Chemical Safety Assessment has been carried out for the mixture.

SECTION 16: Other information

Text of phrases referred to under heading 3:

H225 Highly flammable liquid and vapour.

H315 Causes skin irritation.

H411 Toxic to aquatic life with long lasting effects.

H304 May be fatal if swallowed and enters airways.

H336 May cause drowsiness or dizziness.

H220 Extremely flammable gas.

H280 Contains gas under pressure; may explode if heated.

H319 Causes serious eye irritation.

H332 Harmful if inhaled.

H302 Harmful if swallowed.

	Hazard class and hazard category
	Code
	Description

	Flam. Gas 1
	2.2/1
	Flammable gas, Category 1

	Aerosols 1
	2.3/1
	Aerosol, Category 1

	Press. Gas
	2.5
	Gases under pressure

	Flam. Liq. 2
	2.6/2
	Flammable liquid, Category 2

	Acute Tox. 4
	3.1/4/Inhal
	Acute toxicity (inhalation), Category 4

	Acute Tox. 4
	3.1/4/Oral
	Acute toxicity (oral), Category 4

	Asp. Tox. 1
	3.10/1
	Aspiration hazard, Category 1

	Skin Irrit. 2
	3.2/2
	Skin irritation, Category 2

	Eye Irrit. 2
	3.3/2
	Eye irritation, Category 2

	STOT SE 3
	3.8/3
	Specific target organ toxicity - single exposure, Category 3

	Aquatic Chronic 2
	4.1/C2
	Chronic (long term) aquatic hazard, category 2


Paragraphs modified from the previous revision: 

SECTION 1: Identification of the substance/mixture and of the company/undertaking 

SECTION 2: Hazards identification 

SECTION 3: Composition/information on ingredients 

SECTION 4: First aid measures 

SECTION 7: Handling and storage 

SECTION 11: Toxicological information 

SECTION 14: Transport information 

SECTION 16: Other information 

Classification and procedure used to derive the classification for mixtures according to Regulation (EC) 1272/2008 [CLP]:

	Classification according to Regulation (EC) Nr. 1272/2008
	Classification procedure

	Aerosols 1, H222, H229
	On basis of test data

	Skin Irrit. 2, H315
	Calculation method

	STOT SE 3, H336
	Calculation method

	Aquatic Chronic 2, H411
	Calculation method


This document was prepared by a competent person who has received appropriate training.

Main bibliographic sources:

ECDIN - Environmental Chemicals Data and Information Network - Joint Research Centre, Commission of the European Communities

SAX's DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS - Eight Edition - Van Nostrand Reinold

The information contained herein is based on our state of knowledge at the above-specified date.   It refers solely to the product indicated and constitutes no guarantee of particular quality.

It is the duty of the user to ensure that this information is appropriate and complete with respect to the specific use intended.

This MSDS cancels and replaces any preceding release.

	ADR:
	European Agreement concerning the International Carriage of Dangerous Goods by Road.

	ATE:
	Acute Toxicity Estimate

	ATEmix:
	Acute toxicity Estimate (Mixtures)

	CAS:
	Chemical Abstracts Service (division of the American Chemical Society).

	CLP:
	Classification, Labeling, Packaging.

	DNEL:
	Derived No Effect Level.

	EINECS:
	European Inventory of Existing Commercial Chemical Substances.

	GefStoffVO:
	Ordinance on Hazardous Substances, Germany.

	GHS:
	Globally Harmonized System of Classification and Labeling of Chemicals.

	IATA:
	International Air Transport Association.

	IATA-DGR:
	Dangerous Goods Regulation by the "International Air Transport Association" (IATA).

	ICAO:
	International Civil Aviation Organization.

	ICAO-TI:
	Technical Instructions by the "International Civil Aviation Organization" (ICAO).

	IMDG:
	International Maritime Code for Dangerous Goods.

	INCI:
	International Nomenclature of Cosmetic Ingredients.

	KSt:
	Explosion coefficient.

	LC50:
	Lethal concentration, for 50 percent of test population.

	LD50:
	Lethal dose, for 50 percent of test population.

	PNEC:
	Predicted No Effect Concentration.

	RID:
	Regulation Concerning the International Transport of Dangerous Goods by Rail.

	STEL:
	Short Term Exposure limit.

	STOT:
	Specific Target Organ Toxicity.

	TLV:
	Threshold Limiting Value.

	TWA:
	Time-weighted average

	WGK:
	German Water Hazard Class.


A01029/5

Page n. 
 of 1

